# 3  Data Basis and Preprocessing

## 3.1  Heat Demand Data

As the central component of this project, a district heating network for a fictitious example village comprising 267 households is simulated and optimized over a timespan of one year. The heat load profiles behind it come from hourly smart heat meter readings collected in Aalborg, Denmark, as part of a large-scale monitoring campaign [Schaffer et al., 2022]. The dataset covers 3,021 residential smart heat meters from 2018–2020 at hourly resolution, with contextual metadata on building type and construction year [Schaffer et al., 2022]. 2019 was chosen as the reference year because it has the most complete data coverage and is the last year before the COVID-19 pandemic and the energy-price crisis. Both periods might have influenced demand patterns.

The simulated location Jerrishoe represents a typical northern German village suitable for district heating. It is mainly covered by residential households rather than industrial or commercial consumers. Therefor the Alborg dataset is an appropriate structural proxy since it reflects household consumption.

Nevertheless, Aalborg is not assumed to be equivalent to Jerrishoe. But the dataset provides generic residential district-heating-connected demand dynamics. The resulting mismatch risk is quantified through a temperature and heating-degree-day comparison (Section 3.3) and is explicitly acknowledged as a potential source of bias in the absolute level of heat demand transferred to the model.

## 3.2  Data Processing Pipeline

### 3.2.1  Step 1 – Quality Assurance

Step 1 is a quality-assurance stage. All 3,021 meters are read in for 2019 and merged with the contextual metadata. Duplicate readings within the same hour are averaged. Missing hours are retained as NaN and are deliberately not interpolated at this stage.

Two indicators are derived per meter: annual consumption, computed as the difference between the maximum and minimum cumulative meter reading within 2019. This is robust to gaps in the hourly load column. The peak load comes from the hourly load values where the maximum across the year is taken (NaN-safe). Meters are considered only if their hourly data coverage reaches at least 70%, a threshold established in the smart-meter literature [Schaffer et al., 2022]. Meters with an “unclear” building type, or a negative annual consumption indicating a meter reset, are excluded.

### 3.2.2  Step 2 – Stratified Sampling

The network simulation requires exactly 267 representative hourly load profiles. Rather than generating synthetic ones, real, quality-checked smart meter profiles are drawn from the population using the stratified sampling method [Cochran, 1977]. Meters are first stratified by building type: the amount of profiles allocated to each type is determined proportionally to its population share, using the largest-remainder (Hamilton) procedure to guarantee that the integer counts sum exactly to 267 [Cochran, 1977]. Within each building-type category, meters are further divided into ten quantile bins by annual consumption, and the required number of profiles is drawn randomly according to its population share from each bin. A random seed was set (seed = 42) to ensure full reproducibility [Peng, 2011]. Data gaps of up to six consecutive hours are filled by linear interpolation, corresponding to half a day, a period over which linear interpolation remains physically plausible (reference ?!). Longer outages are left as NaN.

The representativeness of the resulting sample is validated against the full population along three dimensions: the building-type composition, the quantile distribution of annual consumption (P10–P90), and a two-sample Kolmogorov–Smirnov test on annual consumption [Massey, 1951]. The null hypothesis states that sample and population are drawn from the same underlying distribution of annual consumption. The test returned p = 0.99, significantly higher than the conventional 0.05 threshold, indicating no significant distributional difference. In this context, “representative” refers to distributional similarity in building type and consumption, not to a claim of statistical inference to a wider population of DH-connected buildings. The 267 selected profiles are subsequently aggregated into a load time series, forming the basis for the capacity-sizing decisions made in the optimization model [Stoft, 2002].

## 3.3  Temperature Comparison: Aalborg vs. Jerrishoe

To assess whether heat load profiles measured in Aalborg can be transferred to the project location, Jerrishoe, weather conditions in both cities were compared using ERA5 reanalysis data for temperature and solar radiation [Hersbach et al., 2020]. Both the moving-average time series and the temperature duration curves show close alignment between the two locations. This is confirmed quantitatively: Jerrishoe shows a mean annual temperature of 9.9°C versus 9.2°C in Aalborg, with heating degree days of 1,342.5 and 1,491.7 respectively [ISO 15927-6, 2007]. That result states that Aalborg is roughly 11.1% colder than Jerrishoe. Correspondingly, hours below 0°C are more frequent in Aalborg (487 vs. 370 hours).

Because heat demand is temperature-driven, this implies that heat load profiles transferred from Aalborg to Jerrishoe might slightly overestimate demand, leading to a conservative rather than optimistic system dimensioning. As solar radiation, the key driver of PV yield, is comparably close between the two cities (annual full-load hours of 1,095 vs. 1,055 h/kWp). This additionally supports the joint use of Aalborg-based heat demand and Jerrishoe-based PV and weather data, rather than relying on temperature similarity alone.

## 3.4  Electricity, Gas, and PV Data

### 3.4.1  Temperature and PV Generation Profile

The PV generation profile used during in an earlier stage of this project was obtained from [https://www.renewables.ninja/], a web portal that derives PV power time series from underlying reanalysis data. This source was replaced with a self-computed profile based on ERA5 reanalysis data, for following reasons: First, renewables.ninja could only provide a historic time series for Flensburg, not the actual project location and offered very limited control over system assumptions such as module tilt, orientation, or module type. Second, the underlying reanalysis product and processing chain were not transparently documented, making the source unsuitable as a citable primary source. Third, switching to ERA5 allows temperature and radiation data for the PV yield and the heat-pump COP calculation to originate from a single, consistent, peer-reviewed source [Hersbach et al., 2020].

ERA5 is the global atmospheric reanalysis product of the European Centre for Medium-Range Weather Forecasts (ECMWF), distributed via the Copernicus Climate Data Store. It comes with hourly resolution and 0.25° (~28 km) spatial resolution, under a CC BY 4.0 license [Hersbach et al., 2020]. Data were extracted for the grid point nearest to Jerrishoe (54.75°N, 9.25°E; ~13 km from the target coordinates) for the full year 2019. It resulted in 8,760 hourly values of ambient temperature, global, direct and diffuse radiation, and wind speed. Direct normal irradiance and PV output are computed physically using pvlib from module geometry (35° tilt, south-facing) rather than statistically estimated. Further a Carnot-based, temperature-dependent COP is derived for the heat pump. The resulting annual PV yield for Jerrishoe (~1,055 kWh/kWp) is consistent with the comparison in Section 3.3. Relative to SARAH2, a satellite-based product offering higher spatial resolution for solar radiation, ERA5 was deliberately preferred to ensure internal consistency between temperature and radiation within a single source, at the cost of some accuracy in radiation estimates.

At the time of writing, the ERA5-based dataset and the resulting dynamic COP are fully prepared but not yet integrated into the optimisation model, which currently still runs on the previous data sources and a static COP; the distinction between “prepared” and “actively used” is maintained until integration is completed. A comparison run confirms robustness: switching from renewables.ninja to ERA5 leaves the levelised cost of heat unchanged (339.76 €/MWh) and shifts the heat-pump/gas-boiler split by less than 0.1 MWh, with PV full-load hours differing by only 0.4% (1,059 vs. 1,055 h).

### 3.4.2  Electricity and Gas Prices

## 3.5  Data Harmonisation & Cross-Year Consistency

Our input data come from different years and different places, which needs explicit justification when combining them. The heat demand is taken from Aalborg for 2019 [Schaffer et al., 2022], chosen for its high temporal resolution and realistic household-level structure. The locational mismatch risk to Jerrishoe is mitigated through the temperature/heating-degree-day comparison in Section 3.3 and remains subject to sensitivity analysis. Electricity prices are taken from the German day-ahead wholesale market for 2024, chosen to reflect recent price volatility. The risk is a temporal mismatch with the 2019 heat demand data, addressed via a scenario-based approach. The gas price is modelled as a fixed assumption which is acceptable because the gas boiler covers only a share of peak/backup hours. The risk of over- or underestimating its cost is addressed through a sensitivity analysis across low, base, and stress price scenarios.

Both heat demand and electricity prices have strong seasonal structure, which is exactly what makes a direct correlation analysis between them risky — it would likely pick up shared seasonal trends rather than any genuine dependency. So, no causal relationship is claimed here. Both series are treated as independent scenario inputs instead, and their combined effect on the system design is assessed through sensitivity analysis, not correlation-based inference.

## Comments for the Team

This section lists cross-cutting gaps, repetitions, and decisions that affect more than one subsection and should be resolved before the chapter is finalised:

1. Repetition: the Aalborg-≠-Jerrishoe disclaimer appears in full in both 3.1 and 3.5. Recommend keeping the full version in 3.1 and cross-referencing it briefly in 3.5 instead of restating it.

2. The PV/weather year discrepancy (2019 vs. 2024, see note under 3.5) is the single most important open decision in this chapter, since it affects the stated rationale in both 3.4 and 3.5 and possibly the model results themselves.

3. Section 3.4.2 (electricity and gas prices) currently has no source material to draft from — this is the main content gap blocking a complete chapter.

4. Several smaller factual claims (meter-reset mechanism, share of “unclear” meters, randomness of the 20% background failure rate, stability of 2019 as a weather year) are flagged inline above; each needs either a number, a source, or a rewording as an assumption rather than a fact.

5. Citations used throughout follow Literaturquellen_Data_Processing.docx: Schaffer et al. (2022) for the Aalborg dataset, coverage threshold, and interpolation methodology; Cochran (1977) for stratified sampling and the Hamilton allocation; Massey (1951) for the KS-test; Hersbach et al. (2020) for all ERA5 use; ISO 15927-6 (2007) for heating degree days; Stoft (2002) for the load duration curve; Peng (2011) as the reproducibility reference for the fixed random seed. Please double-check in-text citation style (author-date vs. numbered) against your report's overall referencing format, as this draft uses a placeholder bracket style.